Assignment On

"Modern Operating System and Computer Networks"

(Assignment-6)

Submitted by: Madhu boini

2503B09904(MCA)

Submitted to: Dr. Mathivanan D

TCP Congestion Control – Complete Lab Task

Q1. LAB TASK: TCP Congestion Control Simulation

Question:

Using a TCP congestion control simulator (Python or any tool of your choice), simulate and analyze TCP Reno under three different packet-loss conditions:

- 0.1% loss

- 1% loss

- 2% loss

For each case:

- Plot the Congestion Window (cwnd) vs Time for at least 20 seconds of simulation.

- Compute the average throughput (in MSS/second or Mbps).

- Explain how packet loss affects the behavior of TCP Reno, specifically discussing:

  - Slow Start o 	Congestion Avoidance

  - Multiplicative Decrease (cwnd halving)

Compare the three cases and write a short conclusion on how increasing loss impacts network performance.

Code:

# --------------------------------------------------------- # TCP Reno Congestion Control Simulation

# Runs simulations for:

#   - 0.1% loss (0.001)

#   - 1% loss   (0.01)

#   - 2% loss   (0.02)

# Plots cwnd vs time and prints throughput.

# ---------------------------------------------------------

import numpy as np import matplotlib.pyplot as plt import random

# --------------------------------------------------------- # Function to run a single Reno simulation

# --------------------------------------------------------- def run_sim(loss_prob):

RTT = 0.05               # 50 ms RTT     sim_time = 20.0          # run for 20 seconds     mss = 1.0     initial_cwnd = 1.0     ssthresh_init = 64.0     algorithm = "reno"

t = 0.0     cwnd = initial_cwnd     ssthresh = ssthresh_init     time_steps = []     cwnd_trace = []     bytes_acked = 0.0

# Packet sending model     def send_window(cwnd):

segs = int(max(1, round(cwnd)))         lost = False         acks = 0         for _ in range(segs):             if random.random() < loss_prob:

lost = True             else:

acks += 1         return lost, acks

# Main simulation loop (step = RTT)     while t < sim_time:

time_steps.append(t)         cwnd_trace.append(cwnd)

in_slow_start = (cwnd < ssthresh)         lost, acks = send_window(cwnd)         bytes_acked += acks

# TCP Reno update logic         if lost:

ssthresh = max(cwnd / 2, 2)             cwnd = max(cwnd / 2, 1)         else:             if in_slow_start:                 cwnd *= 2             else:

cwnd += 1

cwnd = min(cwnd, 1000)         t += RTT

throughput = bytes_acked / sim_time

# Plot cwnd graph     plt.figure(figsize=(10, 4))     plt.step(time_steps, cwnd_trace, where='post')     plt.xlabel("Time (s)")     plt.ylabel("cwnd (MSS)")

plt.title(f"TCP Reno: loss={loss_prob}, throughput={throughput:.2f} MSS/s")     plt.grid(True)     plt.show()

return throughput

# ---------------------------------------------------------

# Run simulations for 0.1%, 1%, 2% loss

# --------------------------------------------------------- loss_values = [0.001, 0.01, 0.02] results = {}

for lp in loss_values:

print(f"\nRunning simulation for loss = {lp*100}% ...")     results[lp] = run_sim(lp)

# --------------------------------------------------------- # Print final throughput summary

# --------------------------------------------------------- print("\n=== Throughput Summary (20-second simulation) ===") for lp, thr in results.items():

print(f"Loss {lp*100}%  -->  Throughput = {thr:.2f} MSS/s")

Output :